Electric Griddle

User’s Manual

Thank you for purchasing this range of appliance of our company.

Proper attention to preventative maintenance and scheduled maintenance procedures will prolong the life of the appliance.

Warning: Improper installation, adjustment, alteration, service or maintenance can cause property damage, injury or death. Read the installation, operation and maintenance instructions thoroughly before installing or servicing this appliance.

For your safety

Do not store gasoline or other flammable liquids in the vicinity of this or any other appliance.

WARNING: FIRE HAZARD

The area where the appliance is installed must be kept clear of combustibles and flammables. This includes mops, rags, grease, wrapping paper and electrical cords.

Warning

Electrical Grounding Instructions

This appliance is equipped with a three-prong (grounding) plug for your protection against shock hazard and should be plugged directly into a properly grounded three-prong receptacle. Do not cut or remove the grounding prong from this plug. For the appliance whose power input is much larger exceeding a certain value , no plug is used. But the appliance should be grounded safely.

Notice

This appliance is designed for use in non-combustible locations only. Install the appliance on a firm, level, non-combustible surface.

For servicing, a clearance of 6 inches (15cm) from rear of the appliance to wall should be kept.

This appliance is intended for commercial use only. This kind of electric griddle is developed by our company. It absorbs the advantages of similar products home and abroad. It is novel in design and reasonable in structure. It can be used to cook virtually everything from eggs and bacon through to the most elaborate grill. The heavy duty solid steel cooking plates ensure even heat and excellent heat retention. Thermostats are fitted for absolute control and all models have a removable fat drainage below. It is easy to operate and maintain. It is quite good-looking in appearance, which can decorate your working environment. It is an ideal choice for super-markets, western food kitchen, fast-food chains and other food industries.

1. General information

Proper installation, professional operation and consistent maintenance of this appliance will ensure that it gives you the very best performance and a long, economical life.

This instruction manual contains the information needed to properly install the appliance, and to use and care for the appliance in a manner which will ensure its optimum performance. This manual is considered to be a permanent part of the appliance. This manual and supplied instruction, circuit diagrams, schematics, pats breakdown illustrations, notices, warnings and naming plates must remain with the appliance if it is sold or moved to another location. It should be easily accessible for anyone carrying out the installation, servicing, and cleaning.

When using electric appliances, basic safety precautions should always be followed, including the following:

1.1. Read all instructions.

1.2. Do not touch hot surfaces. Use handles or knobs, which may be warm.

1.3. To protect against electric shock, do not immerse cord, plug, or variable heat power cord in water or other liquid.

1.4. Close supervision is necessary when any appliance is used by or near children.

1.5. Unplug from outlet when not in use and before cleaning. Allow to cool before putting on or taking off parts, and before cleaning the appliance.

1.6. Do not operate any appliance with a damaged cord or plug, or after the appliance malfunctions, or has been dropped or damaged in any manner. Call the agent for examination, repair, or adjustment.

1.7. The use of accessory attachments not recommended by the manufacturer may cause injuries.

1.8. Do not use outdoors.

1.9. Do not let cord hang over edge of table or counter, or touch hot surfaces, including the stove.

1.10. Do not place on or near a hot gas or electric burner, or in a heated oven.

1.11. Extreme caution must be used when moving an appliance containing hot oil or other hot liquids.

1.12. Always attach plug to appliance first, then plug cord into the wall outlet. To disconnect, turn any control to “0” then remove plug from wall outlet.

1.13. Use only the power cord provided with this appliance.

1.14. Do not use appliance for other than intended use.

1.15. Do not use without drip tray in place. Clean after each use.

To avoid an electrical circuit overload, do not operate another high wattage appliance on the same circuit with the griddle.

This kind of appliance is equipped with a grounded-type 3-wire cord (3-prong plug). For the appliance whose power input is much larger exceeding a certain value, no plug is used. But the appliance should be grounded safely. This type of plug will only fit into an electrical outlet made for a 3-prong plug. This is a safety feature intended to help reduce the risk of electrical shock. If the plug should fail to fit contact a qualified electrician to replace the obsolete outlet. Do not attempt to defeat the safety purpose of the grounding pin, by modifying the plug in any way.

The length of the cord used on this appliance was selected to reduce the hazards of becoming tangled in, or tripping over a longer cord. If a longer cord is necessary an approved extension cord may be used. The electrical rating of the extension cord must be equal to or greater than the rating of the electric griddle and be a grounded 3-wire cord. Care must be taken to arrange the extension cord so that it will not drape over the countertop or tabletop where it can be pulled on by children or accidentally tripped over.

2. Product Specification

| Model | Voltage | Power Input | Dimension |
| --- | --- | --- | --- |
| GR-400 | 220-240V | 3 kW | 400×540×270 |
| GR-550 | 220-240V | 3 kW | 550×490×270 |
| GR-610 | 220-240V | 6 kW | 610×540×270 |
| GR-760 | 380V | 9 kW | 760×540×270 |

3.  Electrical parts installation

3.1. Make sure the supplied voltage matches the rating voltage on the naming plate.

3.2. Make sure that the appliance is grounded safely. For convenient operation, never pile up the sundry goods near the switch.

3.3 The unit must be placed on an even location. A clearance of 10cm should be kept on the right and left sides. A clearance of 20cm should be maintained between the rear part of the unit and the non-combustible wall.

4. Operation

Before First Use: Wash griddle in hot, soapy water. Rinse thoroughly and dry. The griddle may also be washed in a dishwasher.

  - This appliance is equipped with thermostats, which can set the temperatures at your own requirements. The number of thermostats varies for different models.

  - Always use griddle on a dry, level, heat-resistant surface. Make sure drip tray is in place. Turn the power on, and the green indicator is on. Rotate the thermostat clockwise. Set the temperature at your desired one. Then the yellow indicator is on. It indicates that the heating elements have begun to work. When the set temperature is reached, the power supply would be cut off automatically. The yellow indicator goes off and meanwhile the green indicator is still on, which indicates that the heating elements have stopped working. You may now add the food. Heat may be increased or decreased, depending upon personal preference and the type or amount of food being prepared. When the temperature is lower than the set one, the power supply is on automatically. The yellow indicator is on. The heating elements resumed. The temperature goes up. The appliance repeats in this way to keep the temperature constant.

  - Preheat griddle 5 to 10 minutes until desired temperature is reached. Adjust the temperature for different kinds of food stuff. A satisfactory result can be achieved. When finished cooking, remove food, turn temperature control to 0, and unplug unit.

  - If something unusual happens, stop using it immediately. Call a qualified technician for checking.

NOTES:

- For best results, space foods evenly on the griddle and do not overload the griddle.

- Only use wooden or plastic cooking utensils to prolong the life of the nonstick coating.

- Do not place any pot, pan, or dish on the nonstick griddle surface.

5. Transportation and storage

In the process of transportation, the appliance should be handled carefully, preventing the intense shaking. The packaged appliance should be stored in a well-ventilated area free of caustic air. It cannot be placed in the outside area and should never be put upside down. Keep it dry and dust-free. Do not expose it to direct sunlight. Avoid mechanical shocks and vibrations. If it is for temporary storage, the weather-proof method should be taken.

6. Cleaning and maintenance

6.1. Before cleaning and repairing the unit, cut off the power in case of electric shock. Turn the thermostat to 0 and unplug from outlet.

6.2. Let your griddle cool completely. Remove power cord. Remove drip pan by lifting up and then pulling out. Clean griddle and drip pan with hot, soapy water, using a plastic or nylon scouring pad. Do not use metal scouring pads or abrasive cleansers. This will damage the nonstick coating. Rinse then dry.

6.3 Never use a water jet to clean the appliance. Try to prevent the water flowing into the electrical parts. If so, malfunctions of appliance may occur.

WARNING: To reduce the risk of shock, do not immerse the power cord and plug or any electrical parts in water or other liquid.

7. Notice

  - The voltage and phase of power supply should match that one on the naming    plate.

  - Operators should install suitable switch, fuse protector and three-prong receptacle.

  - The appliance should be grounded safely. No insulation material should be placed between the body and earth. Make sure the appliance is grounded safely to avoid unnecessary electric shock.

  - Before using, be sure that the wire is not loose and the voltage is normal.

  - When the appliance is in use, the body, especially the plate, is of high temperature. Never touch it. You may get burnt.

8. Daily checking

Check the appliance before and after use. Before using, make sure the appliance dose not lean. Be certain that the power cord is not worn out or broken. When in use, make sure no unpleasant smell is present and there is no any unusual noise.

This manual must be read and understood by all persons using or installing this appliance. Contact your local dealer if you have any questions concerning installation, operation or maintenance of this equipment.

IMPORTANT: DO NOT DISCARD THIS MANUAL

This manual is considered to be part of the appliance and is to be given to the owner or manager of the hotel or restaurant, or to the person responsible for training operators of this appliance. It should be retained for future reference. Additional manuals are available from your local dealer.